The Napco "StarLink Connect" Software:  Installation and Setup

With Napco StarLink Connect software and a StarLink radio, you can use the original download software that came with your Honeywell or DSC control panel.  Uploading, downloading and all programming abilities work normally, as the Napco StarLink Connect software is compatible with the Honeywell Compass® Downloader for Windows® Software and the DSC DLS Programming Software.

To install, proceed as follows:

- Run StarlinkConnectSetup.exe to install the Napco StarLink Connect software.

- After the installation process completes, run the software from the desktop icon () or from the Windows Start menu.  When running right click the icon and select “Run as administrator”.

- In the System Tray (usually located at the bottom right side of the Windows Taskbar) click the Napco StarLink Connect icon .

- In the Napco StarLink Connect dialog that appears, click the Settings tab and click the Starlink Authentication button.

- Type the Dealer ID and Password used to log in to the Napco NOC website (www.napconoc.com).  Click OK.

- Close the Napco StarLink Connect dialog by clicking the X in the upper right corner of the dialog. The Exit tab is only used if you want to Suspend or Exit the application; clicking the X in the upper right corner of the dialog will keep the application running in the background.

Using the Napco "StarLink Connect" Software with Honeywell Control Panels

The Napco StarLink Connect software is compatible with the Honeywell Compass® Downloader for Windows® Software.  See www.security.honeywell.com for more information regarding Honeywell control panel compatibility with their Compass software.

Before proceeding, the StarLink Connect SLE radio must be wired to the Honeywell control panel as described in WI2193.  Launch the Honeywell Compass® Downloader for Windows® Software and log in normally.  Create or open an account set up for the Honeywell Panel connected to a Napco StarLink Connect Radio.  Before connecting to the panel, launch the Napco StarLink Connect software by pressing "Alt + S" on the keyboard.  In the popup that appears, click the Compass tab and type the StarLink radio Device ID (printed on the radio PCB sticker) and click OK.

When the Napco StarLink Connect icon in the System Tray turns green with a black "C" in the center (), the Napco StarLink Connect software is ready to connect with the Honeywell Compass software.

When connecting to the panel with the Honeywell Compass software, the Napco StarLink Connect icon in the System Tray turns yellow.  Upload, download and/or program the Honeywell control panel as usual.

To indicate real-time communication activity from the Honeywell Compass software, through the Napco StarLink Connect software to the Honeywell control panel, the "C" in the center of the yellow StarLink icon will toggle between "black on white" and "white on black" ( ).

Upon disconnection, the yellow StarLink icon turns back to green () and waits for another Compass connection.  The last Radio ID used will be retained in memory (RAM) for convenient reconnection.

At any time when not in use you can suspend "StarLink Connect" by right clicking the StarLink icon and selecting "Suspend StarLink Connect" or selecting the "Exit Tab" from any of the StarLink Connect Screens and clicking "Suspend"
 ().

IMPORTANT:  If using a non-StarLink radio to connect to a DSC or Honeywell control panel (using your customary DSC DLS or Honeywell Compass software), ALWAYS click Suspend (described above) to suspend the Napco StarLink Connect software.  If you have to uninstall StarLink Connect software, be sure to “Exit” the application first by selecting the "Exit Tab" (described above).

To summarize the various states of the Napco StarLink Connect software:

Napco StarLink Connect "Suspend" Stopped (initial state)

Napco StarLink Connect "Listening for Compass outgoing connection"

Napco StarLink Connect "Connected with Compass"; during data transfer, yellow icon will toggle between "black on white" and "white on black"  to indicate activity.

Vista Control Panel Wiring

Refer to the “Napco StarLink Connect” WI2193 for more detailed Information.

Example:  (Vista 20P)

- Vista control panel terminal 4 (GND) to StarLink Connect terminal TP8 (GND).

- Vista control panel terminal 6 (Data In) to StarLink Connect terminal TB20 (TX).

- Vista control panel terminal 7 (Data Out) to StarLink Connect terminal TB19 (RX).

- Vista control panel terminal 5 (+) to StarLink Connect terminal TB1 (+).

- Vista control panel terminal 4 (-) to StarLink Connect terminal TB2 (-).

Vista Control Panel Programming

Refer to the Honeywell control panel programing instructions for the exact programming locations and other information.  In general, program as follows:

Example:  (Vista 20P)

- Enter Program Mode (Installers Code) (800).

- Enable "Long Range Radio Output" (*29) = (1).

- Change "Dynamic Signaling Priority" to "Long Range Radio First" - (*55) = (1).

- Enable Device 20: KP5 (*193) = (1,0).

- Ensure that the "Sub Account Number" (*43) and the "CSID" (if used) are identical in both the control panel and the account. To View "Sub Account Number" - (#43) display (N1,N2,N3,N4).

- Exit Program Mode (*99).

Using the Napco "StarLink Connect" Software with DSC Control Panels

The Napco StarLink Connect software is compatible with the DSC DLS Programming Software.  See www.dsc.com for more information regarding DSC control panel compatibility with their DLS software.

Before proceeding, the StarLink Connect SLE radio must be wired to the DSC control panel as described in WI2193.  Launch the DSC DLS Programming Software and log in normally.  Create or open an account set up for the DSC control panel connected to a Napco StarLink Connect Radio.  Before connecting to the DSC control panel, launch the Napco StarLink Connect software by pressing "Alt + S" on the keyboard.  In the popup that appears, click the DLS tab and type the StarLink radio Device ID (printed on the radio PCB sticker) and click OK.

When the Napco StarLink Connect icon in the System Tray turns green with a black "D" in the center (), the Napco StarLink Connect software is ready to connect with the DSC DLS Programming Software.

When connecting to the control panel with the DSC DLS Programming Software, the Napco StarLink Connect icon in the System Tray turns yellow.  Upload, download and/or program the DSC control panel as usual.

To indicate real-time communication activity from the DSC DLS Programming Software, through the Napco StarLink Connect software to the DSC control panel, the "D" in the center of the yellow StarLink icon will toggle between "black on white" and "white on black" ( ).

Upon disconnection, the yellow StarLink icon turns back to green () and waits for another DSC DLS Programming Software connection.  The last Radio ID used will be retained in memory (RAM) for convenient reconnection.

At any time when not in use you can suspend "StarLink Connect" by right clicking the StarLink icon and selecting "Suspend StarLink Connect" or selecting the "Exit Tab" from any of the StarLink Connect Screens and clicking      "Suspend" ().

IMPORTANT:  If using a non-StarLink radio to connect to a DSC or Honeywell control panel (using your customary DSC DLS or Honeywell Compass software), ALWAYS click Suspend (described above) to suspend the Napco StarLink Connect software.  If you have to uninstall the StarLink Connect software, be sure to “Exit” the application first by selecting the "Exit Tab" (described above).

To summarize the various states of the Napco StarLink Connect software:

Napco StarLink Connect "Stopped" (initial state)

Napco StarLink Connect "Listening for DLS Programming Software outgoing connection"

Napco StarLink Connect "Connected with DLS Programming Software"; during data transfer, yellow icon will toggle between "black on white" and "white on black"  to indicate activity.

DSC Panel Wiring using the PC-Link Harness

Refer to the “Napco StarLink Connect” WI2193 for more detailed Information.

Example: (PC1616)

- DSC control panel terminal (PC-Link 1) to StarLink Connect terminal TB20 (TX)

- DSC control panel terminal (PC-Link 2) (GND) to StarLink Connect terminal TB8 (GND)

- DSC control panel terminal (PC-Link 3) (GND) to StarLink Connect terminal TB8 (GND)

- DSC control panel terminal (PC-Link 4) to StarLink Connect terminal TB19 (RX)

- DSC control Panel terminal (YEL) to StarLink Connect terminal TB12 (CLK)

- DSC control Panel terminal (GRN) to StarLink Connect terminal TB13 (DATA)

- DSC control Panel terminal (RED) to StarLink Connect terminal TB1 (+)

- DSC control Panel terminal (BLK) to StarLink Connect terminal TB2 (-)

DSC Panel Programming

Refer to the DSC control panel programing instructions for the exact programming locations and other information.  In general, program as follows:

Example: (PC1616)

- Enable "T-Link"  [*] [8] [Installer Code] [382] [5] [#] [#]

- Enable "Double Call Enabled" [401]-1

Additional Settings and Features

Info Button:  Will show info on the “StarLink Connect” Radio.

- Radio Version

- Micro 2 Version

- Manufacturer Type Setting

- Panel Type

- Last Check-in

- Signal Strength

Check Radio/Panel Type:  Check that Radio is a “StarLink Connect” Radio with the Correct “Panel Type” before going into “Ready to Connect” mode.

PC-Security Support: When setting the “Ready to Connect” for a “StarLink Connect” radio the PC-Security Code prompt will come up if a “PC-Security Code” has been used. Will remember the last code used until you exit the application from the “System Tray”.